Driver Drowsiness Detection

Using Computer Vision & AI

Shani Patel

Electronics & Communication Engineering

November 2025

# Abstract

This project presents a real-time driver drowsiness detection system built with Python, OpenCV, and MediaPipe. The system uses a standard webcam to continuously monitor facial landmarks and computes two key behavioral metrics — Eye Aspect Ratio (EAR) and Mouth Aspect Ratio (MAR) — to detect signs of fatigue and yawning before they cause a road accident.

The system runs entirely on consumer hardware (a laptop or Raspberry Pi), requires no wearable sensors, and alerts the driver in real time. Testing across multiple users and lighting conditions showed a detection accuracy of over 91%, with low latency and a false positive rate under 8%.

# 1. Introduction

Drowsy driving is responsible for a significant proportion of road accidents globally. Unlike aggressive driving or distracted phone use, fatigue builds gradually and is often unnoticed by the driver until it is too late. Traditional solutions — such as physiological sensors or vehicle telemetry — are either intrusive, expensive, or too slow to react.

This project takes a different approach: using a camera already present in most modern vehicles or laptops, it watches the driver's face in real time. By measuring how open the eyes are and detecting yawning patterns, the system can warn a driver within seconds of the onset of drowsiness.

## 1.1 Why This Approach?

Vision-based detection has several practical advantages over alternatives:

- No wearable hardware — the driver doesn't need to attach anything to their body

- Works on standard laptops and embedded boards like Raspberry Pi

- All processing happens on-device, so no internet connection or cloud dependency is needed

- Can be updated with improved AI models without changing any hardware

# 2. How It Works

The system follows a five-stage pipeline from video capture to alert output:

## 2.1 Video Capture

A webcam captures live video at 24–30 frames per second. Each frame is passed through the pipeline for analysis. The system works with USB webcams, built-in laptop cameras, and the Raspberry Pi Camera Module.

## 2.2 Facial Landmark Detection

Google's MediaPipe Face Mesh model extracts 468 precise 3D facial landmarks from each frame. This works reliably under different lighting conditions, head angles, and with accessories like glasses. The model runs fast enough for real-time operation even on modest hardware.

## 2.3 Eye Aspect Ratio (EAR)

EAR is a geometric ratio computed from six landmark points around each eye. When the eye is fully open, EAR stays above a baseline value (~0.25–0.30). As the eye closes, EAR drops. If it stays below the threshold for more than 40 consecutive frames (~1.5 seconds), a drowsiness alert is triggered.

EAR  =  (A + B) / (2 × C)

Where A and B are vertical distances between eyelid landmarks, and C is the horizontal distance across the eye.

## 2.4 Mouth Aspect Ratio (MAR)

MAR works on the same geometric principle but applied to the mouth. When a driver yawns, the mouth opens wide, raising the MAR value above 0.6. If this is sustained for 15 or more frames, a yawning alert is triggered. This acts as an early-warning signal, often appearing before full drowsiness sets in.

## 2.5 Alert System

When a drowsiness or yawning event is detected, the system:

- Displays a bold on-screen text overlay (red for drowsiness, blue for yawning)

- Shows live EAR and MAR values for monitoring

- Can be extended to trigger an audio buzzer or GPIO signal on embedded hardware

# 3. System Architecture

The system is designed in clearly separated modules, making each part easy to test, upgrade, or replace independently:

| Module | Description |
| --- | --- |
| Video Input | Webcam feed captured via OpenCV VideoCapture at 24–30 FPS |
| Preprocessing | Frame resized and converted to RGB for MediaPipe input |
| Landmark Extraction | MediaPipe Face Mesh outputs 468 3D facial landmark coordinates |
| Feature Calculation | EAR and MAR computed from relevant landmark subsets |
| State Inference | Consecutive-frame counters determine drowsy/yawn/alert states |
| Alert Output | On-screen overlay; extendable to audio or GPIO hardware alerts |

# 4. Implementation

## 4.1 Technology Stack

- Python 3.7+ — core programming language

- OpenCV — video capture and frame rendering

- MediaPipe — facial landmark detection (468 points per frame)

- NumPy — numerical computation for EAR/MAR geometry

## 4.2 Key Parameters

These values were tuned through testing to balance sensitivity and false alarm rate:

| Parameter | Value | Purpose |
| --- | --- | --- |
| EAR Threshold | 0.25 | Below this = eye closing |
| MAR Threshold | 0.60 | Above this = mouth open (yawn) |
| Drowsy Frame Count | 40 frames | ~1.5 sec of closed eyes triggers alert |
| Yawn Frame Count | 15 frames | ~0.5 sec of open mouth triggers alert |

## 4.3 Running the System

To run the project, install the dependencies and execute the script:

pip install opencv-python mediapipe

python drowsiness_detection.py

# 5. Results & Performance

The system was tested across 8 volunteers in varied lighting conditions — daylight, fluorescent office light, and low-light evening settings — both with and without glasses. Key performance metrics are summarised below:

| Metric | EAR Only | EAR + MAR | Literature Range |
| --- | --- | --- | --- |
| Accuracy | 86–92% | 91–96% | 72–99% |
| Precision | 88% | 93% | 70–94% |
| Recall | 85% | 95% | 71–98% |
| F1 Score | 86% | 94% | 76–97% |
| False Positive Rate | 12% | 5–8% | 6–22% |
| Latency (ms) | 55–100 ms | 32–55 ms | 30–300 ms |
| Processing Speed | ~27 FPS | ~25 FPS | 10–30 FPS |

The combined EAR + MAR approach outperformed the eye-only baseline on all metrics, while still maintaining real-time processing speed. Performance degraded by only 2–3% when tested on users wearing glasses, confirming the robustness of the MediaPipe landmark model.

# 6. Limitations

The current version has the following known constraints:

- Requires a roughly front-facing camera angle; extreme side profiles reduce landmark accuracy

- Performance drops in very low light or with sudden bright glare (e.g., direct sunlight)

- Opaque sunglasses block the eye region entirely, making EAR-based detection unreliable

- No physiological or vehicle-based data is used, so complex edge cases may be missed

- Long-term use may lead to alert fatigue if thresholds are not personalised per user

# 7. Future Work

Several improvements are planned for future versions:

- Add head pose estimation to detect nodding, which is another early sign of drowsiness

- Introduce LSTM-based temporal modelling to catch slow-onset fatigue more reliably

- Build a per-user calibration step to personalise EAR and MAR thresholds

- Add an audio alert (buzzer) for embedded deployment on vehicles

- Integrate with vehicle OBD-II data for context-aware alerting (speed, time of day)

- Expand testing to a larger and more diverse dataset across age groups and ethnicities

# 8. Conclusion

This project demonstrates that a reliable, low-cost drowsiness detection system can be built using freely available tools — a webcam, Python, OpenCV, and MediaPipe. By combining EAR and MAR metrics with temporal frame logic, the system accurately distinguishes genuine fatigue from normal blinking or brief expressions, achieving over 91% accuracy in real-world tests.

The solution is hardware-agnostic, privacy-friendly (no data leaves the device), and easily deployable on both laptops and embedded systems like Raspberry Pi. It has clear potential for integration into vehicle safety systems and represents a practical step toward reducing drowsy-driving accidents.

Shani Patel  —  November 2025